Model: AP2402

How to set up as repeater

1. Go to router setup page, 192.168.1.1, login in as guest

2. At setup page, select wireless – wireless setting – radio mode, select repeater

3. Perform AP scan and select the wireless router SSID you want to connect, connect & save.

4. If you have a security password setup for your original router, please setup wireless – wireless security, please make sure all setting are the same as your previous router, save all settings.

5. Go to DHCP option – select “disable”, save and reboot. You should be able to access internet from the router.

Notes: Please make sure you change and save all settings before go to next page, reboot after all settings are changed.

The steps are also showed on the user manual page 44-46.